- Manufacturers: Subject to comply with requirements of bathroom accessories by one of the following:

- A & J Washroom Accessories, Inc.

- American Specialties, Inc.

- Bobrick Washroom Equipment, Inc.

- Bradley Corporation.

- General Accessory Manufacturing Co.

- McKinney/Parker Washroom Accessories Corp.

- Items to be supplied and installed by contractor:

- Mirror-glass ASTM C 1036, Type I, Class 1, 6.0 mm thick.

- Coat hooks. (chrome plate).

- Grab bars. (stainless steel).

All items tamper proof designed.

  - Items to be supplied by owner & installed by contractor:

- Soap dispensers.

- Paper towel dispensers.

- Toilet paper dispensers.

- Electric hand dryers not permitted.

- Coat hooks shall be installed by contractors on all interior partitioned stall areas.

- Framed mirrors:  Secure to walls in concealed, tamper-resistant manner with special hangers, toggle bolts, or screws.  Set units level on walls according to manufacturers instructions.

- Install grab bars to withstand a downward load of at least 250 lb.   According to ASTMF 446.

- Diaper changing station shall be surface-mounted, mildew-resistant, molded Polyethylene body that folds horizontally against wall when not in use; projects not more than 4 inches (100 mm) from wall when closed; and engineered to support a minimum of 250-lb (113-kg) static weight when opened.  Provide unit with pneumatic shock-absorbing operating mechanism and built-in dispenser for sanitary liners.  Diaper changing stations supplied & installed by contractor.

  - Approved Manufacturers or equal:

- American Infant Care Products Inc.

- American Specialties, Inc.

- Brocar Products, Inc.

- General Accessory Manufacturing Co. (GAMCO).

- Koala Corporation

- Safe-Strap Company, Inc.